Test board Architecture and Implementation for neuron hardware accelerator engine

Tel Aviv University

Electrical Engineering Final Project 2024-2025

Version 0.6

28.05.2025

Yuval Desalto

Ariel Richter

Table of Contents

Overview	3

Theoretical Background	4

Requirements Definition	5

Performance Requirements	5

Validation Requirements	6

Top Block Diagram	8

Pre-Silicon Validation (FPGA-Based Testing – RTL Simulation)	9

Post-Silicon Validation (Fabricated DUT Testing – Real Hardware Validation)	9

Pre silicone	11

FPGA and DUT Integration Architecture	11

Components and Signals of FPGA-DUT integration	12

Signals and data transfer Between Subsystems	13

Data Bus Width and Multi-Cycle Transfers	13

Test environment	14

Test environment Components and Signals	15

Workflow of the Test Environment	17

Test environment RTL System Overview	18

Generator Diagram	19

Components and Signals of the Generator	19

Generator Workflow	20

Validator Diagram	21

Components and Signals of the Validator	21

Validator Workflow	22

DUT Block Diagram	23

DUT Components and Signals	23

DUT Workflow	26

MAC Diagram	28

Components and Workflow of MAC unit	28

MAC Workflow	29

System Workflow: End-to-End Testing Process	30

Step-by-Step Execution of the Testing Workflow	30

On-Chip Memory Structure and Test Format	32

Managing Button Bounce in the Reset Mechanism	34

Methodology:	34

How This Solves the Problem?	35

Post silicone	36

Hardware Setup for Post-Silicon Validation	36

Pin Placement	36

The diagram below shows the final planned pin placement for the DUT	36

Pinout Description	37

System-Level Schematic: DUT Integration and Power Routing	37

Appendix	39

# Overview

The test board architecture is a key component of the testing framework for the MAC core, which serves as the Device Under Test (DUT) in this project. This architecture is designed to enable functional validation, performance evaluation, and reliability testing of the DUT during the pre-silicon validation and initial hardware testing stages. It provides a structured and modular approach to managing data flow, control signals, and test scenarios to ensure the MAC core functions correctly under various conditions.

At the center of the architecture is the FPGA, which acts as the primary platform for running test cases and simulating real-world scenarios. The FPGA includes buffers for data input and output, comparators for result validation, and a control unit that manages the testing process. This design ensures proper data communication between the FPGA and the DUT while enabling precise control of test parameters.

The test board integrates with external controllers or PCs to provide additional computational resources and manage data logging and analysis. This integration enhances flexibility and scalability, allowing the architecture to support different testing stages and future system requirements.

The architecture is built with modularity in mind, allowing individual functional blocks to be developed, tested, and optimized independently. This ensures that the testing system remains adaptable and capable of meeting evolving project needs while maintaining the accuracy and efficiency necessary for validating advanced semiconductor devices.

# Theoretical Background

Hardware testing systems (Test Boards) are a critical component in the development and verification of advanced chips, particularly in projects based on deep learning principles and neural networks. This project focuses on developing a dedicated Test Board for testing a computational accelerator (MAC Core) based on the perceptron model. Designing and building such a system requires a deep understanding of the perceptron structure, data processing methodologies, and FPGA-based hardware verification. FPGA (Field Programmable Gate Array) platforms are central to this process, serving as the primary testing environment. Their reconfigurable architecture allows for the implementation of custom test benches that manage data input, output, and real-time validation of the DUT. This hardware-based approach ensures precise timing analysis, real-time error detection, and the ability to test under hardware-specific conditions, offering significant advantages over traditional software simulations. By leveraging FPGA technology, the system enables comprehensive validation, ensuring early detection of functional and integration issues while supporting a reliable development process.

A key aspect of the computational accelerator being tested is its reliance on the perceptron model, a foundational concept in machine learning and neural networks. The perceptron’s role and functionality are explained below to provide context for the DUT’s operation.

#### The Perceptron Model

The perceptron is a foundational concept in machine learning, mimicking the behavior of a single neuron in a neural network. It operates by performing a linear combination of input values, weighted by their respective coefficients, and adding a bias term. Mathematically, this can be expressed as:

y=i=1nwi∙xi+b

where wi are the weights, xi are the inputs, and b is the bias term.

The perceptron’s output is determined by an activation function, which applies a threshold to the result of the linear summation. The function is defined as:

y= 1  ;if y≥Threshold0  ; otherwise

In this project, the perceptron serves as the central computational unit within the MAC Core. Its purpose is to process input data efficiently, producing binary outputs that indicate whether certain conditions are met, such as recognizing patterns or calculating results in line with predefined thresholds.

# Requirements Definition

The Test Board is designed to validate the functionality and performance of the computational accelerator (MAC Core), serving as the Device Under Test (DUT). The system requirements are defined to ensure reliable operation, compatibility with the FPGA platform, and the ability to achieve high testing coverage with minimal latency. These requirements encompass functional, performance, and validation metrics, ensuring the system meets the demands of pre-silicon and hardware-based testing stages.

## Performance Requirements

- Clock Frequency:
The Test Board must operate at a clock frequency of 50 MHz. This frequency was selected based on the DE10-Lite FPGA’s operational specifications, ensuring compatibility with the clock management resources and the overall timing constraints of the board. The DE10-Lite supports clock frequencies within this range while maintaining stable and reliable operation. Operating at this frequency ensures that the design meets the timing requirements for accurate data processing and synchronization with the Device Under Test (DUT). This value was specifically chosen to optimize performance while ensuring the stability and reliability of the system under typical operating conditions.

- FPGA Resource Utilization:
The Test Board must limit its utilization to 50% of the FPGA’s available logic resources. This limitation was established to ensure that sufficient headroom remains for future development, debugging, or modifications that may arise as the project progresses. The DE10-Lite FPGA, equipped with 50,000 Logic Elements (LEs), is designed to accommodate designs of moderate complexity. By capping the utilization at 50%, we can prevent potential routing congestion and maintain timing stability across the design. This conservative allocation also ensures that any unforeseen requirements or features can be integrated into the system without requiring significant rework or impacting the system's performance.

- Power Consumption:
The Test Board must operate with a power consumption limit of 5W. This value was determined based on the specific usage capabilities of the DE10-Lite FPGA for our implementation. According to the DE10-Lite user manual, the power delivery system of the board is capable of reliably supplying this level of power, which includes both static (idle) and dynamic (operational) power consumption. By adhering to this limit, we ensure that the FPGA operates within its thermal and electrical constraints, avoiding overheating or power delivery issues. Additionally, maintaining this power limit aligns with efficient energy use and ensures that the board can support stable, long-term operation under typical conditions for our project requirements.

These requirements were carefully defined to balance the FPGA’s capabilities with the needs of the project, providing a robust foundation for current functionality while allowing room for future development.

## Validation Requirements

To ensure the correctness and reliability of the DUT, the validation process is divided into three key phases: pre-silicon simulation, pre-silicon RTL prototyping, and post-silicon validation. Each phase targets different aspects of verification, from logical correctness to real-world system behavior. Additionally, real-time error reporting is an essential feature that supports debugging and validation across all stages but is not considered a separate validation phase.

#### Pre-Silicon Simulation

#### Pre-silicon simulation is the first stage of validation, where the DUT is tested in a software-based simulation environment. This phase focuses on verifying the logical correctness of the DUT before synthesis. Functional simulations are conducted at the RTL level to ensure that the DUT behaves as expected under different input scenarios. This process allows early detection of logical errors, misalignment with specifications, and potential inefficiencies in the design.

#### Pre-Silicon RTL Prototyping

To bridge the gap between simulation and real hardware testing, FPGA-based prototyping is used at the RTL level. The Test Board must enable the DUT to run on an FPGA platform, allowing for more accurate performance analysis, early debugging, and testing under real-world conditions. Unlike simulations, FPGA-based validation allows the design to run in actual hardware, exposing potential timing-related issues, memory access conflicts, and interface mismatches that may not appear in purely software-based simulations.

#### Post-Silicon Validation

Once the DUT has been fabricated or integrated into the FPGA platform, post-silicon validation is performed. This stage involves real-time functional and performance testing to identify any remaining issues that were not detected in pre-silicon phases. The system must support extensive real-time testing, including stress testing, functional corner cases, and hardware/software co-validation. The goal is to ensure that the implemented hardware meets all design specifications and operates correctly under realistic conditions.

#### Error Reporting (Applicable Across All Phases)

#### Error reporting is a fundamental aspect of the validation process. it plays a supporting role across pre-silicon simulation, RTL prototyping, and post-silicon validation. The Test Board must provide a robust real-time error detection and reporting mechanism, capturing details such as failed test cases, timing violations, and output mismatches. The error reporting system should be capable of detecting both transient and persistent errors, allowing efficiently trace back to root causes.

#### Flexibility and Scalability

The Test Board must support updates to the test environment, including modifications to test scenarios, additional DUT components, and changes in input/output configurations. This ensures adaptability to evolving project requirements and future scalability.

# Top Block Diagram

The top block diagram represents the overall architecture of the project, structured into two distinct testing phases: pre-silicon validation and post-silicon validation. These phases ensure a comprehensive verification process, starting from RTL-level simulations to final hardware validation. The system integrates an FPGA-based test environment, an external DUT board for post-silicon testing, and a PC/controller, allowing for seamless data communication and evaluation of the MAC core at different development stages. This architecture provides a flexible and scalable testing framework, enabling precise debugging, performance analysis, and functionality verification across both stages.

Figure 1 – Pre silicone and Post silicone Top Block Diagrams

## Pre-Silicon Validation (FPGA-Based Testing – RTL Simulation)

During the pre-silicon phase, the MAC core is tested using an FPGA-based prototype before fabrication. This stage focuses on RTL verification and functional testing.

- The MAC core (RTL) is synthesized and implemented inside the DE10-Lite FPGA, along with the test environment.

- The test environment is responsible for generating input stimuli, handling execution, and validating outputs within the FPGA.

- Data is transferred between the PC/controller and FPGA via a UART interface, allowing test execution and real-time debugging.

- The PC/controller manages test execution, sending test vectors and analyzing results to ensure correctness before hardware fabrication.

- This stage helps identify logical errors, performance bottlenecks, and design flaws in a fully controlled simulation environment.

## Post-Silicon Validation (Fabricated DUT Testing – Real Hardware Validation)

Once the MAC core has been fabricated into a physical chip, testing transitions to the post-silicon phase. This phase ensures that the real hardware implementation operates as expected under real-world conditions. Unlike pre-silicon testing, the MAC core is no longer inside the FPGA but instead exists on a separate DUT board, requiring external validation.

- The DE10-Lite FPGA now functions as a test environment, generating test stimuli and validating outputs received from the fabricated MAC core on the DUT board.

- The DUT board contains the fabricated MAC core, which processes the test inputs and produces outputs for verification.

- Data transfer between the FPGA and DUT board occurs through a GPIO connector, ensuring correct signal exchange between the test system and the DUT.

- The PC/controller remains the central management unit, retrieving results from the FPGA and analyzing the DUT's performance.

- This stage helps identify hardware-related issues, fabrication defects, timing mismatches, and integration challenges that may not have been detected during pre-silicon simulation.

### PC/Controller

- Acts as the central control unit for managing and monitoring both pre-silicon and post-silicon testing processes.

- In pre-silicon testing, it sends test vectors to the FPGA and receives simulation results for analysis.

- In post-silicon testing, it retrieves test results from the DUT through the FPGA.

- Provides an interface for test configuration, execution, and real-time debugging.

- Uses USB 3.0 for high-speed data transfer between the PC and FPGA.

### Interfaces

- UART (Data In/Out) – FPGA to PC

  - Enables bidirectional communication between the FPGA and the PC/controller.

  - Used in both pre-silicon and post-silicon testing for logging test results and sending control signals.

- GPIO Connector – FPGA to DUT Board

  - Handles data exchange between the FPGA test environment and the DUT.

  - Ensures proper transmission of test vectors and retrieval of processed results.

  - The exact connector type will depend on the final DUT board selection.

# Pre silicone

# FPGA and DUT Integration Architecture

The system architecture is based on an FPGA and integrates various components to create a comprehensive and self-contained testing environment. At its core, the architecture includes a host computer that communicates with the system via a USB Blaster interface, enabling efficient data transfer and FPGA programming. A communication pathway is established using the FT245 component, which converts USB signals to UART, and the MAX II CPLD, which manages UART communication and supports JTAG access. A clock generator provides synchronized clock signals to drive the entire system, ensuring timing consistency across all components.

Figure 2 - FPGA and DUT architecture

The FPGA, based on a MAX 10 logic array, hosts the full testing environment, including the control unit, test data generator, output validator, and on-chip memory blocks used to store input vectors and expected results. The Device Under Test (DUT) is integrated directly within the FPGA and interacts with these modules through a shared data bus and control signals. This architecture eliminates the need for external SDRAM, relying solely on low-latency on-chip memory to manage test data, which simplifies data flow and improves system reliability. The result is a compact, modular, and flexible testing platform capable of supporting complete validation cycles entirely within the FPGA.

## Components and Signals of FPGA-DUT integration

#### Host

The Host represents the external computer or controller used to communicate with the FPGA-based system. It sends commands, configurations, and data to the system via a USB Blaster interface.

#### USB Blaster

The USB Blaster serves as a bridge between the host computer and the FPGA system. It enables data transfer and programming of the FPGA through a USB interface.

#### FT245

#### The FT245 is a USB to UART bridge that converts USB signals into UART protocol. It facilitates efficient data flow between the host and the internal system, serving as a key component in communication.

#### MAX II (240)

#### The MAX II CPLD serves as a bridge between the UART interface and the system, managing communication and signal flow. It supports both UART and JTAG protocols for programming and debugging the FPGA.

#### Clock Generator

#### The clock generator provides the necessary clock signals to synchronize the operations of the system. It generates the DRAM_CLK signal, which is critical for operating the SDRAM and ensuring proper data transfer timing.

#### Logic Array (MAX 10)

The MAX 10 FPGA is the main programmable logic component in the system, hosting key subsystems:

- Test Environment:

  - Control Unit: Manages the testing process, synchronizes data flow, and coordinates communication with the DUT.

  - Generator: Sends input patterns (pixel-weight pairs) to the DUT.

  - Validator: Compares the DUT’s single-bit output against the expected result.

  - On-Chip Memory: Internal memory used to store test data, thresholds, expected results, and validation outputs.

- Device Under Test (DUT):

  - Contains the MAC-based core and activation function.

  - Receives pixel-weight inputs and produces a single-bit binary output.

### Reset Button

The Reset Button is a physical push button that reinitializes the system. When pressed, it issues a reset signal across all components, clearing internal states and memory contents and returning the system to a defined start state. This ensures a clean restart before running each test and is essential during development and debugging. The reset also triggers initialization of internal counters and logic without requiring external memory loading.

## Signals and data transfer Between Subsystems

- DATA_BUS[15:0]: A 16-bit one directional bus used for data exchange between the test environment and the DUT.

- output: Carries the processed output data from the DUT to the test environment.

- output_ready: Indicates that the DUT has completed processing, and its output is ready for validation.

- chip_sel: Activates or enables the DUT during operations.

- wr_en: A write enable signal that initiates write operations within the DUT.

This diagram outlines a highly organized architecture where the host interfaces with the FPGA system via a USB link, data flows seamlessly through the various components, and the logic array performs testing and validation of the DUT with high precision.

## Data Bus Width and Multi-Cycle Transfers

The system utilizes a 16-bit wide data bus, a design decision made to optimize resource efficiency since the majority of operations involve transferring 16-bit data. However, for operations requiring 22-bit data, such as loading the threshold value, data transfers are conducted over two consecutive clock cycles.

During two cycle operations:

- In the first clock cycle, bits 0–15 are written to the target destination.

- In the second clock cycle, bits 16–22 are written, ensuring complete data transfer.

This two-cycle approach efficiently supports operations requiring 22-bit data while aligning with the design decision to use a 16-bit data bus. The choice of a 16-bit bus was made because the majority of system operations inherently involve transferring 16-bit data, making this width optimal for resource utilization and performance. While threshold loading require handling 22-bit data, the two-cycle transfer mechanism ensures compatibility without compromising the system’s overall efficiency.

# Test environment

Figure 3 – Test environment Diagram

The test environment is designed to facilitate a controlled and efficient validation process for the DUT by generating test data, validating its outputs, and managing intermediate memory operations. It consists of key components: a generator that controls sending test vectors to the DUT, a validator that checks correctness of the DUT’s output, an on-chip memory block that stores both test inputs and expected result, and a control unit that coordinates all actions. The system data transfer operates over a data bus and uses a global clock to ensure synchronization. This architecture supports precise sequencing, minimal latency.

## Test environment Components and Signals

#### Control Unit

The control unit acts as the central coordinator of the test environment. It controls the sequencing of test execution, signals other components to start operations, and manages memory addressing for data and result storage.

Signals and data:

- Inputs:

  - start: Triggers the start of a new test cycle.

  - clk: Synchronizes the operations across the test environment.

  - reset: Reset signal from pushbutton.

- Outputs:

  - start_run: Triggers operations in the generator.

#### Generator

The generator produces test data and manages its flow into the DUT. It navigates input data from the on-chip memory and sends it to the DUT along with control signals.

Signals and data:

- Inputs:

  - clk: Synchronizes the generator’s operations with the control unit.

  - reset: Resets the generator’s internal states.

  - start: Initiates data generation.

  - val_done: indicated validation process is done and we can lower chip_sel signal.

- Outputs:

- address_BUS[6:0]: Specifies the memory address for read from on-chip memory

- chip_sel: Selects the appropriate chip for data transfer (also responsible for resetting the chip components when chip_sel goes from low to high).

- wr_en: Enables writing test data to the DUT.

- rd_en: enables reading data from on chip memory.

  - Validator

The validator compares the DUT's output with expected result stored in memory. It ensures the DUT's functionality by signaling success or failure based on the comparison.

Signals and data:

- Inputs:

  - clk: Synchronizes the validator’s operations with the control unit.

  - reset: Resets the Validator’s internal states.

  - output_ready: Indicates that the DUT finished its calculation.

  - expected_output: Expected result read from memory.

  - output:  1-bit output from the DUT.

- Outputs:

- val_done: indicates validation process is done.

- Pass/fail: 1-bit result indicating success or failure

  - On-Chip Memory

he on-chip memory serves as the system’s data store, preloaded with test vectors and expected results. It operates in close coordination with the generator, ensuring low latency data access during testing.

Signals and data:

- Inputs:

  - address[6:0]: Specifies the memory location for read operations.

  - rd_en: Enables reading data from memory.

  - clk: Synchronizes memory operations.

  - Reset: resets on_chip_memory components .

- Outputs:

  - DATA_BUS[15:0]: Provides test data to the DUT.

  - Expected_res: expected result to the validator.

  - Reset and Start Module

The reset + start block receives the physical pushbutton signal (KEY0) and generates clean reset and start pulses. It uses internal debouncing logic to ensure stable signals are sent to the control unit.

Signals and data:

- Inputs:

- KEY0: Active-low pushbutton after debouncing from the board.

- Outputs:

- reset: Active-low system reset signal.

- start: One-cycle start pulse initiating the test.

## Workflow of the Test Environment

#### Initialization Phase

After the FPGA is programmed, the system awaits a manual press on the reset button (KEY0). The reset + start module debounces the physical signal and generates a clean, active-low reset signal that propagates to all components in the test environment.

Once the button is released, a single-cycle start pulse is issued to the control unit, triggering the beginning of the test sequence.

#### Test Execution Start

The control unit receives the start pulse and initiates the test by sending a start_run signal to the generator. This begins the process of reading input data from the on-chip memory.

#### Data Generation Phase

The generator activates chip_sel to enable the DUT, then issues rd_en signals to the on-chip memory, while sequentially generating address[6:0] values to retrieve pixel-weight pairs.

Although the generator controls the timing, the data path is direct: test data flows straight from memory to the DUT via the data_bus[15:0]. For each valid transfer, the generator asserts wr_en to signal the DUT to latch the input.

This process continues until all 64 input pairs + Threshold value have been sent.

#### Processing Phase

The DUT receives pixel-weight pairs and processes them through its internal pipeline, applying multiplication, accumulation, and thresholding.

Once the DUT completes processing, it asserts the output_ready signal and places the 1-bit result on its output port.

#### Validation Phase

Upon detecting output_ready, the validator uses the expected result from the on-chip memory (via a separate expected_res port), compares it to the DUT’s output, and determines a 1-bit pass/fail result.

The validator then asserts the val_done signal, notifying the generator that validation has completed.

#### Result Indication Phase

The validator’s 1-bit pass/fail result is routed directly to LED9 and LED8 on the DE10-Lite board:

- If LED9 turns on, the test passed (output matches expected).

- If LED8 turns on, the test failed (mismatch detected).

#### Idle Phase

Once the validator raises val_done and the result has been displayed, the system enters an idle state. It remains ready to process a new test sequence upon the next reset and start pulse.

## Test environment RTL System Overview

Figure 4 - RTL block diagram generated by Quartus

The following RTL diagram, automatically generated by the Quartus synthesis tool, provides a visual overview of the full system architecture as implemented on the FPGA. It illustrates the interconnections between all major components of the test environment, including the clock generator, debounce logic, control unit, generator, on-chip memory, DUT, and validator.

This RTL representation confirms that each module is instantiated and connected correctly, and allows verification of data/control signal flow prior to FPGA deployment.

# Generator Diagram

The generator is a key module in the FPGA-based testing environment. It is responsible for orchestrating the transfer of input data from the on-chip memory to the Device Under Test (DUT). Operating under the control of the system clock and a start signal from the control unit, the generator issues read requests to the memory and controls the timing of data delivery to the DUT. It generates sequential memory addresses through an internal address counter and manages control signals such as rd_en, chip_sel, and wr_en to ensure proper synchronization and activation of the DUT. While the test data flows directly from memory to the DUT via the shared data bus, the generator determines when and how the DUT should latch each data item, enabling precise control over the test execution process.

Figure 5 - Generator Diagram

## Components and Signals of the Generator

#### Components:

- Control Logic

  - Manages the overall operation of the generator.

  - Sends control signals to both the on chip memory and the DUT to ensure correct data transfer.

  - Responsible for asserting the required signals to the DUT, such as wr_en and chip_sel to enable data movement.

- Address Counter

  - Generates sequential addresses for reading test data from the on-chip memory.

  - Advances the address on each clock cycle after receiving the start signal.

  - Operates in sync with the control logic to ensure that memory access is orderly and timed correctly.

### Signals and data

#### Inputs:

- reset: Resets the generator to its initial state, clearing counters and control signals.

- clk: The clock signal used to synchronize the generator's operations with the system.

- start: A signal from the control unit that initiates the data transfer process.

- output_ready: received from DUT to give feedback about the duration of time that chip_sel needs to stay high.

#### Outputs:

- address_BUS[6:0]: Address for reading data from on-chip memory

- rd_en: Read enable signal to access data from the on chip memory.

- chip_sel: Chip select signal to enable the DUT for receiving data (needs to stay high until one cycle after output_ready is received).

- wr_en: Write enable signal to load the retrieved data into the DUT.

Note: The data itself flows directly from the on-chip memory to the DUT via the DATA_BUS[15:0]. The generator does not drive or receive this bus directly.

## Generator Workflow

When the generator receives a start signal from the control unit, it begins a structured sequence to deliver test vectors to the DUT. The address counter generates a new read address on every clock cycle and sends it to the on-chip memory. At the same time, the control logic asserts rd_en to request data from that address.

Although the generator does not handle the data path directly, it manages the timing of all related control signals. As the memory outputs each test word onto the DATA_BUS[15:0], the generator issues wr_en to the DUT, instructing it to capture the data. The chip_sel signal is kept high throughout the data transfer and lowered only after the validator indicates (val_done) that the test result has been processed.

This mechanism ensures that data is retrieved and delivered to the DUT in the correct order and with precise control, enabling reliable and repeatable test execution.

# Validator Diagram

Figure 6 - Validator Diagram

The validator is a critical component in the test environment, responsible for verifying the correctness of the DUT’s binary output. It compares the single-bit result generated by the DUT to the expected value stored in memory. The validator operates in real time: once the DUT signals output_ready, the validator performs the comparison and generates a pass/fail signal that is directly routed to LEDs on the board. This mechanism enables fast feedback and reliable identification of incorrect computations.

## Components and Signals of the Validator

Components-

- Control Logic

  - Coordinates the validation process based on system clock and reset.

  - Monitors the output_ready signal and triggers the comparison.

  - Asserts the val_done signal to notify the generator when validation is complete.

- Output Comparator

  - Receives both the actual DUT output and the expected result from memory.

  - Performs a bitwise comparison.

  - Generates a 1-bit pass/fail result indicating whether the DUT output is correct

### Signals and data-

#### Inputs:

- reset: Resets the validator, clearing all internal states in the control logic.

- clk: Synchronizes the operations of the validator with the system clock.

- output_ready: Indicates that the DUT has finished processing, and its output is available for validation.

- output: The data output from the DUT that needs to be validated.

- expected_output: The pre-stored expected results fetched from memory for comparison.

#### Outputs:

- result: 1-bit signal indicating whether the DUT output matches the expected result.

- val_done: Signals that validation is complete and the generator can proceed.

## Validator Workflow

Once the DUT finishes processing and asserts output_ready, the validator’s control logic initiates the comparison process. It receives the 1-bit output from the DUT and the corresponding expected output from the on-chip memory. These values are forwarded to the output comparator, which performs a direct comparison.

If the values match, the comparator asserts the pass/fail output accordingly (e.g., logic high for pass). The result is routed to the system’s LED indicators to give immediate visual feedback. After comparison, the validator asserts val_done, signaling that the system can return to idle or proceed to the next test.

# DUT Block Diagram

### This diagram illustrates the internal architecture of the DUT, which is designed to process pixel-weight pairs and produce final outputs for validation. The DUT integrates an IO block, control unit, input and weights memory blocks, a MAC unit, an activation function, and an output memory block. Each component operates in synchronization, with the control unit orchestrating data flow and processing across the pipeline.

Figure 7 - DUT diagram

## DUT Components and Signals

#### 1. IO Block

The IO block serves as the main interface for data transfer between external systems and the DUT. It facilitates initialization by receiving pixel, weight, and threshold data, and provides real-time access to intermediate results and final outputs. The IO block ensures seamless communication with the DUT's internal components.

Inputs:

  - DATA_BUS[15:0]: Transfers input data (pixels, weights, and thresholds) from external systems to the DUT.

  - wr_en: Enables writing input data into the DUT’s memory.

  - chip_sel: Selects specific components or memory blocks for data transfer.

  - clk: The clock signal used to synchronize the generator's operations with the system.

Outputs:

  - output: Provides the processed final results to external systems.

  - output_ready: Indicates the completion of the DUT's processing pipeline and readiness of output data.

#### 2. Control Unit

The control unit coordinates the operation of all subsystems within the DUT, including memory access, MAC unit activation, and threshold application in the activation function. It generates control signals to manage data flow and ensures that each component operates in proper synchronization.

Inputs:

  - clk: Synchronizes the DUT's operations.

  - wr_en: Enables writing input data into the DUT’s memory.

  - chip_sel: Selects specific components or memory blocks for data transfer.

Outputs:

  - wr_data_ptr: Specifies the address in memory for data writing.

  - rd_data_ptr: Specifies the address in memory for data reading.

  - mul_mem_en: Activates the MAC unit’s multiplication operation.

  - ac_mem_en: Enables the MAC unit’s accumulator to store intermediate results.

  - rst_mem: Resets the Output Memory, and Accumulator to their initial states before computation begins.

  - threshold_ready: Indicates readiness to sample and apply the threshold value.

  - output_ready: Signals that the DUT has finished processing.

#### 3. Input Memory

The input memory stores pixel data received during initialization. During computation, it supplies pixel data to the MAC unit for processing.

Inputs:

  - wr_data_ptr: Specifies the memory address for writing pixel data.

  - rd_data_ptr: Specifies the memory address for reading pixel data during computation.

  - clk: Synchronizes memory operations.

  - rst_mem: Clears the memory contents before starting a new computation.

#### 4. Weights Memory

The weights memory stores the weight values corresponding to the input pixels. These weights are retrieved during computation for processing by the MAC unit.

Input Signals:

  - wr_data_ptr: Specifies the memory address for writing weight data.

  - rd_data_ptr: Specifies the memory address for reading weight data during computation.

  - clk: Synchronizes memory operations.

  - rst_mem: Resets the memory contents before a new computation cycle.

#### 5. MAC (Multiply-Accumulate) Unit

The MAC unit performs the core arithmetic operations, multiplying pixel data with weight values and accumulating the results. It operates under the control of the control unit and passes its final output to the activation function.

Inputs:

  - Pixel data: Received from the input memory.

  - Weight data: Received from the weights memory.

  - mul_mem_en: Enables the multiplication operation.

  - ac_mem_en: Enables the accumulation operation.

  - clk: Synchronizes the MAC operations with the system.

  - rst_mem: Resets the accumulator before starting a new computation.

Outputs:

  - mac_output_data: The accumulated result sent to the activation function.

#### 6. Activation Function

The activation function applies a threshold to the accumulated result from the MAC unit, generating a binary output.

Inputs:

  - MAC_output: Data from the MAC unit to be processed.

  - threshold_ready: Indicates that the threshold value is ready to be applied.

  - threshold: The predefined threshold value used for comparison.

Outputs:

  - Processed data: Sent to the output memory for storage.

#### 7. Output Memory

The output memory stores the processed data from the activation function. It serves as a buffer, allowing the IO block to retrieve results for external validation or further processing.

Inputs:

  - Data: Received from the activation function.

  - rst_mem: Resets memory for new computations.

  - clk: Synchronizes memory operations with the DUT.

  - rd_en: Enables reading data for external systems.

Outputs:

  - output: Sent to the IO block for external retrieval.

## DUT Workflow

- Initialization Phase

  - The FSM within the control unit manages the initialization by orchestrating memory write operations.

  - Pixel data and weight values are transferred from the IO block to the input memory and weights memory using the wr_data_ptr signal.

  - The threshold value is also received from the IO block and stored in the threshold register within the activation function.

  - The rst_mem signal is asserted to reset all internal memories and prepare the system for a new computation cycle.

- Data Fetch Phase

  - The control unit issues the rd_data_ptr signal to sequentially read pixel data from the input memory and weight values from the weights memory.

  - The data is fed into the MAC unit in the correct order for computation, ensuring alignment between the pixel and weight pairs.

- Processing Phase

  - The MAC unit performs element-wise multiplication of pixel and weight pairs using its multiplier.

  - The results are zero-padded and passed to the accumulator via the adder to calculate a cumulative sum.

  - The control unit generates the mul_mem_en and ac_mem_en signals to enable the multiplication and accumulation processes, ensuring proper synchronization of operations.

- Thresholding Phase

  - After all pixel-weight pairs have been processed, the cumulative result from the MAC unit is sent to the activation function.

  - The threshold_ready signal is asserted, enabling the threshold register within the activation function to sample the threshold value from the data bus during the final cycle of computation.

  - The activation function compares the accumulated result with the stored threshold value and generates a binary output (1 or 0) based on whether the result meets or exceeds the threshold.

- Output Storage Phase

  - The binary output from the activation function is written to the output memory under the control of the FSM using the wr_en signal.

  - The output memory stores the final results, making them available for retrieval by external systems.

- Result Retrieval Phase

  - Once the computation is complete, the output_ready signal is asserted, signaling that the final processed results are available.

  - The IO block retrieves the output data from the output memory and sends it to external systems for validation or further use.

- Idle Phase

  - After all computations are completed and the result is retrieved, the control unit transitions the DUT into an idle state, preparing the system for the next operation cycle.

# MAC Diagram

Figure 8 - MAC diagram

The MAC (Multiply-Accumulate) unit is a fundamental component within the DUT, responsible for executing the arithmetic operations required for processing pixel-weight pairs. This unit performs element-wise multiplication followed by accumulation of the results, which are then passed to subsequent processing stages. The MAC unit operates in a synchronized manner, ensuring that intermediate results are retained across multiple cycles, allowing for efficient and precise calculations.

## Components and Workflow of MAC unit

#### Components:

- Multiplier

  - Receives input pixel and weight values, performing element-wise multiplication.

  - Outputs a 16-bit result, which is forwarded to the next stage.

- Register

  - Temporarily holds the multiplication result produced by the multiplier.

  - Facilitates stable data flow to the adder, synchronized with the system clock.

  - Applies zero-padding to the 16-bit multiplication result to match the 22-bit format required by the subsequent stages.

- Adder

  - Receives the multiplication result from the register and adds it to the current accumulated value from the accumulator.

  - Produces a 22-bit summation result for the accumulation stage.

- Accumulator

  - Stores the intermediate and final summation results.

  - Provides the current accumulated value to the adder for continuous summation.

  - Outputs the final accumulated result as a 22-bit value.

### Control Signals

- mul_mem_en

  - Enables the register to store the multiplier’s output.

  - Ensures that the multiplication result is sampled and held for addition.

- ac_mem_en

  - Enables the accumulator to sample and store the intermediate summation result.

  - Facilitates the continuous accumulation process by updating the stored value after each addition.

- rst_mem

  - Resets the accumulator to its initial state, clearing any previous summation results.

## MAC Workflow

- At the start of the computation cycle, the MAC unit receives pixel and weight values through the img input and Weight input ports.

- The multiplier performs element-wise multiplication of the inputs, generating a 16-bit result.

- The multiplication result is stored in the register, which stabilizes the data for the next computation step.

- The adder receives the multiplication result and adds it to the accumulated sum retrieved from the accumulator.

- The updated summation is stored in the accumulator, ensuring the retention of intermediate results across cycles.

- Once all pixel-weight pairs are processed, the final accumulated result is made available at the MAC_output port for further processing.

### Output

The MAC unit generates a 22-bit accumulated result, which is made available at the MAC_output port. This result can be utilized for further stages, such as activation or threshold comparison. The real-time accessibility of the output also allows for monitoring and debugging during the computation process.

This structured and efficient workflow ensures precise processing of pixel-weight pairs and robust arithmetic operations within the DUT.

# System Workflow: End-to-End Testing Process

Figure 9 – Full workflow diagram

The system is designed to validate the Device Under Test (DUT) by executing predefined test cases, comparing each output against an expected result, and reporting success or failure via onboard LEDs. The workflow is divided into three coordinated stages: host-side preparation, FPGA-side test execution, and host-side post-processing.

## Step-by-Step Execution of the Testing Workflow

#### Test Preparation on the Host (PC)

- The host system prepares the test file using a Python script.

- Each test case includes:

  - 64 pixel-weight pairs (each 16-bit word contains one 8-bit pixel and one 8-bit weight)

  - A threshold value (16-bit)

  - An expected binary result (1 bit, stored in a 16-bit word)

- The host initializes communication with the FPGA via JTAG or Control Panel.

- The generated test file is written directly to the on-chip memory of the FPGA.

- Once loaded, the test data resides entirely within the FPGA, eliminating the need for external memory or runtime communication with the host.

### System Initialization

- The user presses the reset button (KEY0) on the FPGA board.

- A debounced, active-low reset signal is asserted and distributed to all components: Control Unit, Generator, Validator, DUT, and On-Chip Memory.

- When the button is released, a one-cycle start pulse is generated by the reset_and_start module.

- The start signal triggers the control unit, which begins orchestrating the test flow.

#### Data Generation Phase

- The control unit sends a start_run signal to the generator, initiating data transmission.

- The generator:

  - Enables chip_sel, activating the DUT.

  - Starts generating sequential memory addresses using its internal address counter.

  - For each address:

    - Asserts rd_en to read from on-chip memory.

    - Sends wr_en to the DUT to indicate data should be latched.

- The 16-bit data flows directly from memory to the DUT via the shared DATA_BUS[15:0].

- This process continues until all 64 input pairs + Threshold value are transmitted.

#### Processing Phase

- The DUT processes the incoming pixel-weight pairs:

  - Multiplies and accumulates values via its MAC unit.

  - Applies the threshold using its internal activation function.

- Once computation is complete, the DUT:

  - Outputs the binary result (output, 1 bit)

  - Asserts output_ready to signal that the result is valid

### Validation Process

- The validator detects output_ready and initiates the comparison process.

- The output comparator compares the actual DUT output with the expected result.

- If the values match, the validator sets pass/fail = 1 (indicating success); otherwise, it sets pass/fail = 0.

- The validator asserts val_done to notify the generator that validation has completed.

#### 6. Output Display Phase

- The 1-bit result is routed directly to the DE10-Lite board’s LEDs: If pass/fail = 1, LED9 turns on - test passed. If pass/fail = 0, LED8 turns on - test failed

- This provides immediate, visual feedback without host intervention.

### 7. System Reset and Next Test Cycle

- After the val_done signal is received, the generator de-asserts chip_sel, completing the test.

- The system enters an idle state, awaiting the next reset and start pulse to begin a new test.

- If additional test vectors are loaded (manually or automatically), the process can be repeated without reprogramming the FPGA.

# On-Chip Memory Structure and Test Format

The on-chip memory serves as the primary storage for all test-related data used during FPGA-based validation. It is implemented as an internal memory array inside the FPGA, with a fixed structure that supports efficient access by both the generator and the validator.

#### Memory Format and Layout

Each test case is stored as a contiguous memory block, with a fixed size and layout. The memory block contains:

- Pixel-Weight Pairs (64 words)

  - Each 16-bit word stores one 8-bit pixel and one 8-bit weight.

  - These values are paired sequentially in the format: {pixel, weight}.

  - Total: 64 words = 64 × 16 bits.

- Threshold Values (2 words)

  - A fixed 22-bit threshold is stored across two 16-bit words.

  - The first word contains the lower 16 bits of the threshold.

  - The second word contains the upper 6 bits, padded with leading zeros.

  - Total: 2 words = 32 bits used (with 10 unused bits in the second word).

- Expected Result (1 bit)

  - The expected binary classification result (0 or 1) for the test.

  - This value is stored in a dedicated internal register (expected_reg) and is output alongside memory reads.

In total, each test block occupies 66 memory words of 16 bits each, stored in addresses mem[0] to mem[65].

#### Python-Based Test File Generation

To automate the creation of test vectors, a custom Python script is used. This script generates:

- Random or predefined pixel-weight matrices.

- A valid threshold value.

- The expected binary output, based on internal MAC + threshold logic.

- The script then converts the data into 16-bit words and writes them in a format that matches the on-chip memory layout exactly.

These test blocks can be:

- Written directly into memory using the Quartus Control Panel or a JTAG interface.

- Embedded in Verilog code for simulation purposes using initial blocks.

#### Memory Access

- During testing, the generator reads the pixel-weight and threshold values using a sequential address pattern (address[6:0]), while asserting rd_en.

- The validator accesses the expected output result via the expected_out signal, which remains stable across the test.

- The DUT receives the test data directly from the data_out line, which is driven by the memory.

This structure ensures that the entire test dataset is localized within the FPGA, enabling high-speed testing without external memory access and facilitating deterministic, cycle-accurate validation.

The figure below illustrates the internal Verilog implementation of the on_chip_memory module. It highlights the structure of a preloaded test case containing 64 pixel-weight pairs, 2 threshold words, and 1 expected binary result.

Figure 10 - on_chip_memory file

# Managing Button Bounce in the Reset Mechanism

When using a physical push button, the mechanical contacts inside the button can cause a phenomenon called button bounce. This occurs because when the button is pressed or released, the contacts do not settle immediately. Instead, they can oscillate between an open and closed state for a few milliseconds. This bouncing effect causes the button's signal to transition multiple times between logical 0 and 1, potentially leading to unintended behaviors in digital circuits.

Figure 11 – Button Bounce for active high button

In our case, since the reset button is active-low and is used to reset the system, any unintended multiple transitions caused by bouncing could result in erratic or unintended resets. Similarly, when releasing the button, residual bouncing could result in the reset signal being incorrectly deactivated multiple times or too soon.

Debouncing Method: Using a Counter-Based Approach

To address the bouncing issue, we implemented a debouncing mechanism in the FPGA using a counter-based approach.
This ensures that the reset signal is only recognized as valid when it has been stable for a sufficient number of clock cycles.

## Methodology:

#### 1. Signal Stability Verification with a Counter

- The signal from the physical button (KEY0) is sampled on every clock cycle.

- A counter tracks how long the button signal remains unchanged.

- If the signal remains stable for a predefined number of clock cycles (e.g., corresponding to 10ms), it is considered valid.

- If the signal fluctuates before reaching the required duration, the counter resets, ensuring only a stable input is processed.

#### 2. Stable Reset Activation and Deactivation

- When the button remains pressed (logical 0) for the full duration of the counter, the system recognizes a valid reset signal.

- When the button remains released (logical 1) for the full duration, the system registers that the reset has been deactivated.

- Any rapid fluctuations caused by bouncing are ignored, as only a signal that maintains stability over multiple clock cycles is considered valid.

## How This Solves the Problem?

This debouncing method ensures:

Stable Reset Activation:

- The reset is activated only when the button has been consistently pressed (logical 0) for the required duration.

- This prevents false resets caused by transient bouncing during a press.

Stable Reset Deactivation:

- The reset is deactivated only when the button has been consistently released (logical 1) for the required duration.

- This prevents unintended state transitions caused by bouncing during release.

Noise Filtering:

- Any rapid fluctuations in the button signal due to mechanical bouncing are ignored.

- The counter ensures that only a signal that remains stable over multiple clock cycles is processed.

# Post silicone

# Hardware Setup for Post-Silicon Validation

To support smooth integration of the fabricated chip on the PCB, the design uses the QFP (Quad Flat Package) form factor. This package was selected due to its mechanical robustness, accessibility, and compatibility with standard surface-mount assembly processes. The QFP offers leads on all four sides of the package, which simplifies routing of I/O signals and power rails across multiple directions on the board.

The selected variant includes 44 leads with a 0.8 mm pitch, ensuring sufficient pin count for our data bus, control signals, and power interfaces, while maintaining a compact physical footprint. The exposed pad and grounded leads also help improve thermal performance and electrical stability during high-speed signal operation.

This packaging choice provides a practical balance between integration density and testability, making it well-suited for the post-silicon validation stage, where direct access to all signal lines is essential.

## Pin Placement

## The diagram below shows the final planned pin placement for the DUT

Figure 12 – Top package Pin Placement

## Pinout Description

- Data I/O Bus signals (Bus[0] to Bus[15]) are placed consecutively along the bottom and right sides of the package to allow for compact and uniform routing of the 16-bit bus.

- Control signals such as  wr_en, chip_sel, output_ready, and output are grouped on the left side to ensure clean timing alignment and simplified interface with the FPGA.

- The clock input (clk) is located at the top-left corner, isolated from data lines to minimize signal interference and jitter.

- Power domains are clearly separated for electrical stability:

  - VDD_IO (1.8V) and GND_IO provide power and ground for the I/O interface.

  - VDD_core (0.9V) and GND_core supply the internal logic of the chip.

- Several pins are intentionally marked as Not Connected, allowing layout flexibility and future expansion if needed.

This pin configuration was chosen to support clear signal organization, good power integrity, and ease of integration with the FPGA board and level shifter. It reflects the final layout constraints while preserving accessibility for debugging and measurement during post-silicon validation.

# System-Level Schematic: DUT Integration and Power Routing

As part of the preparations for the post-silicon validation phase, that will take place next year, we created a schematic that illustrates the expected physical integration of the fabricated chip on the test board. This schematic serves as a clear communication tool between the FPGA validation team and the chip design team, showing how signals and supply voltages are routed to the DUT. The focus is on representing the signal flow and power delivery rather than detailed pin-level schematics.

The following figure presents a system-level schematic that outlines the physical wiring and voltage domains surrounding the DUT during post-silicon operation. It highlight component placement, power sourcing, and data path routing between the DUT, level shifters, and the external control interface.

Figure 13 – Dut integration and Power Routing

On the right side of the system is the DUT, which operates with two separate power domains: 1.8V (Vdd_io) for the I/O interface, and 0.9V (Vdd_core) for internal logic. The DUT receives a 16-bit wide Data_BUS, a CLK signal, and several control inputs including rd_en, wr_en, and chip_sel. It also provides output signals such as output and output_ready.

A level shifter, located at the center of the diagram, converts signals from the 3.3V logic domain (used by the FPGA and GPIO interface) down to the appropriate levels required by the DUT - 1.8V for I/O and 0.9V where necessary for internal logic interfacing. All bidirectional or output signals from the DUT also pass back through the level shifter to be compatible with the FPGA's 3.3V domain.

The DC/DC converter receives a 3.3V input and supplies two output rails: 1.8V for the DUT's I/O domain and 0.9V for the core logic. These are distributed to both the DUT and the level shifter.

On the left side, the GPIO connector serves as the interface with the FPGA. It delivers all incoming control signals, clock, and data lines at 3.3V logic level. These signals are routed through the level shifter to ensure compatibility with the lower-voltage DUT.

# Appendix

Tasks to Be Completed-

### Unknown yet-