### SDRAM Data Management and Host Communication

#### 1. Host Process Workflow

The host is responsible for preparing and transferring test files to the FPGA via the Control Panel software, as well as retrieving processed results. The process follows these steps:

- Test File Preparation:

  - The host generates a structured test file containing input matrices, weight values, threshold values, and expected results.

  - The file format aligns with the SDRAM memory structure for efficient data processing.

- Data Transfer to SDRAM:

  - Using the Control Panel software, the host writes multiple test cases sequentially into SDRAM.

  - Each test case occupies 153 bytes in SDRAM, where most of the space is allocated for input data, and only 2 bits represent the test environment computed result.

- Test Execution on FPGA:

  - The FPGA first transfers test cases from SDRAM to the on-chip memory, which acts as an intermediate buffer.

  - The on-chip memory holds a smaller set of test cases, ensuring faster access during processing.

  - Test cases are loaded into on-chip memory in batches, ensuring a continuous execution flow without needing direct access from SDRAM during processing. Once a batch is processed, a new batch is loaded from SDRAM into on-chip memory, allowing efficient memory utilization.

  - From the on-chip memory, test data is distributed to the relevant components within the test environment for execution.

  - After processing, the computed 2-bit result is stored back in SDRAM at a predefined location.

- Retrieving Results and Processing:

  - The host reads only the 2-bit test results, keeping the original test inputs in memory.

  - A validation script compares the retrieved results with expected values and generates a pass/fail report highlighting any discrepancies.

#### 2.  SDRAM Data Structure

The memory is organized to store test cases efficiently:

- Each test case occupies 153 bytes, which includes:

  - Input matrices and weight values

  - Threshold values

  - Expected results

  - 2-bit test result, stored at the end of the allocated space for each test

- The FPGA control unit uses read and write pointers to sequentially access and update data, ensuring continuous processing.

#### 3. FPGA Reconfiguration for SDRAM Updates

The FPGA does not require reprogramming when uploading new test data. The architecture allows dynamic loading and retrieval of test cases without modifying the bitstream. New datasets can be written to SDRAM at any time using the Control Panel software.

#### 4. Host Data Retrieval and Organization

After processing, the FPGA writes the 2-bit test result for each test case back to SDRAM. The host:

- Reads only the computed results while maintaining the test inputs in memory.

- Runs a validation script to analyze results and generate a pass/fail report.

#### 5. Semi-Automated Testing Execution

The process is semi-automated, requiring manual intervention before each test batch:

- Before running new test cases, the reset button on the FPGA must be pressed to restore a stable system state.

- Once reset, the host transfers test data, triggers execution, and retrieves results automatically.

- While each test sequence runs autonomously, a manual reset is required between batches.

### 6. Randomization and Targeted Testing Implementation

To ensure comprehensive validation, the testing framework includes both randomized testing for broad coverage and targeted edge-case testing for verifying specific conditions. Each method is implemented differently to achieve effective verification.

#### Randomized Testing Implementation

- The host software generates randomized input matrices and threshold values before writing them to SDRAM.

- The randomization process is implemented using a Python script that ensures:

  - Values remain within the valid numerical range.

  - Weight distributions vary to simulate different network behaviors.

  - Different threshold values are tested to observe activation function performance.

- The randomized data follows the same format as regular test cases to ensure compatibility with the FPGA processing flow.

- After each batch, results are retrieved, and the process repeats for multiple random scenarios.

#### Targeted Edge-Case Testing Implementation

- These tests focus on specific conditions that require explicit validation.

- The host maintains a predefined set of test cases, including:

  - Boundary values for minimum and maximum inputs.

  - Stress scenarios with highly skewed weight distributions.

  - Known failure points where the MAC unit or activation function may exhibit edge-case behavior.

- These test cases are manually defined and loaded into SDRAM to ensure controlled testing conditions.

- The results from targeted cases are compared against expected outputs to verify that specific functionality meets design specifications.

By combining automated randomization with controlled edge-case testing, the system ensures broad test coverage while validating critical failure points in a structured manner.