## Python Test Generation and Validation System

### Part A: Test Generation

- Purpose

The Python test generation tool is designed to automatically create a complete set of test cases for validating a single-neuron MAC-based architecture implemented on FPGA. Each test includes a matrix of pixel inputs, weights, a threshold value, and expected results.

- Process Overview

- Test Definition
The generator defines a variety of test scenarios:

  - Functional tests

  - Randomized tests

  - Walking 1s / Walking 0s patterns

  - Edge case tests (e.g., all-zero, all-maximum values, invalid thresholds)

- Expected Result Calculation
For each test, the tool computes the MAC (dot product of pixel and weight matrices). This value is then compared against a 22-bit threshold to determine the expected single-bit output.

- Unified Test File Format
All tests are compiled into a single file named full_tests.hex using the following format:

  - Line 0: Number of tests (16-bit hexadecimal)

  - For each test:

    - Header line: ABCD

    - 64 lines of input pairs: each line contains one pixel (8-bit) and its corresponding weight (8-bit), combined into 16-bit hex

    - 2 lines for the 22-bit threshold (split into two 16-bit values)

    - 2 lines for the expected MAC result (also 22-bit split into two) and expected binary output (0 or 1).

- Individual Test Files
In addition to the unified file, the tool also generates separate files for each test (inputs, weights, threshold, and expected output), which are saved and compressed into a ZIP archive named all_tests_hex.zip.

### Part A.5: Test File Loading and Result Extraction via Control Panel

Once the test file has been generated, the user loads it into the FPGA system using the Intel Control Panel utility. The process includes:

- Test Loading
The full_tests.hex file is loaded into the SDRAM on the FPGA via the Control Panel. The SDRAM acts as the input buffer that holds all test data to be processed by the FPGA.

- Test Execution
Once loaded, the on chip memory on the FPGA reads the test data from SDRAM and runs each test through the MAC-based neuron logic.

- Result Capture
After all tests are complete, the FPGA writes one 16-bit result per test into a results region in SDRAM. The user then uses the Control Panel again to read back this result memory space and export it to a file named validation_results.

- File Location
The exported result file (validation_results) is saved into the generated_tests/ directory, ready for validation by the Python script.

### Part B: Validation of Results

- Purpose

After the tests are streamed to the FPGA and the results are exported from SDRAM, the Python validation script analyzes the results to determine whether the DUT behaved correctly for each test.

- Format of the Validation Results File

Each line in the validation_results file represents one test and contains a single 16-bit bin value. Only the 2 least significant bits are used for validation:

- Bit 0 indicates whether the single-bit output matched the expected result

- Bit 1 indicates whether the MAC value matched the expected MAC

- Validation Procedure

- The Python validator opens the ZIP file all_tests_hex.zip and extracts test names based on the _inputs.hex files.

- The order of the extracted test names is assumed to match the order of the results in the validation_results file.

- Each 16-bit result is parsed, and the 2 least significant bits are evaluated.

- For each test, the validator reports:

  - whether the test passed

  - or, in case of failure, whether the MAC comparison, the single-bit comparison, or both failed

- A summary is printed to the terminal, and any failed test names are written to a file named failed_tests.txt, including the reason for failure.

- Example Output

[✓] Functional_Typical passed

[✗] Random_Test_4 failed (Single-bit mismatch)

[✗] Walking_0s_7 failed (MAC mismatch, Single-bit mismatch)

=== Validation Summary ===

Total tests   : 100

Passed tests  : 97

Failed tests  : 3

Some tests failed — see 'failed_tests.txt' for details.