Test plan

Test Bord for VLSI- single neuron hardware accelerator engine

Tel Aviv University

Electrical Engineering Final Project 2024-2025

Version 0.5

28.5.2025

Yuval Desalto

Ariel Richter

Table of Contents

Introduction	3

Objectives	3

interfaces	3

Testing Approach	3

How This Approach Guarantees Full Coverage?	4

Testing plan	5

Test Scenarios	7

Detailed Explanations for Test Scenarios	7

1.	Randomized Input Test	7

2.	Maximum Value Test	8

3.	Minimum Value Test	8

4.	Invalid Threshold Test	9

5.	Threshold Sensitivity Test	9

6.	Interrupted Data Flow Test	9

7.	Sparse Input Test	10

8.	Reset During Operation Test	10

9.	Delayed Output Validation Test	10

# Introduction

This Test Plan outlines a comprehensive approach to validate the functionality, performance, and reliability of the MAC Core, the Device Under Test (DUT). The focus is on testing the external behavior of the system by observing its inputs and outputs under a variety of conditions. The plan includes detailed scenarios to ensure that the DUT meets its design specifications, functions robustly under edge cases, and handles errors gracefully. By simulating real-world operating conditions and extreme scenarios, this Test Plan ensures that the system is ready for integration into larger applications and meets high standards of reliability and efficiency.

# Objectives

- Validate the core functionality of the MAC Core in accordance with its design specifications.

- Confirm correct operation under maximum, minimum, and randomized input values.

- Assess the DUT’s behavior when facing data interruptions, delayed output usage, and mid-stream resets.

- Ensure the DUT maintains correctness and system integrity when exposed to threshold-edge cases or sparse input patterns.

# interfaces

This Test Plan evaluates the external behavior of the MAC Core by providing controlled inputs and observing its binary output. The test scenarios target a wide range of validation aspects including normal operation, boundary conditions, input irregularities, and operational disruptions. Through this, the plan verifies that the DUT maintains functional correctness, error resilience, and data integrity under diverse testing conditions.

# Testing Approach

- Randomized Testing:

  - Generate randomized pixel-weight pairs and threshold values.

  - Compare DUT output to a software model to validate correctness.

  - This type of testing provides broad coverage and exposes corner cases not covered by directed tests.

  - Integration with Test Case: Randomized Input Test.

- Directed Value-Based Testing

  - Use predefined inputs with known maximum and minimum values.

  - Validate DUT’s correctness in handling edge conditions like all-zeros and all-ones inputs.

  - Integration with Test Cases: Maximum Value Test and Minimum Value Test.

- Threshold Boundary Testing

  - Provide inputs that produce cumulative sums near the threshold to test sensitivity.

  - Ensure binary output toggles correctly when crossing threshold boundaries.

  - Integration with Test Case: Threshold Sensitivity Test.

- Interrupted and Delayed Operation Testing

  - Assess system behavior when data flow is paused mid-stream or when output reading is delayed.

  - Verify system resumes correctly and produces valid results.

  - Integration with Test Cases: Interrupted Data Flow Test and Delayed Output Validation Test

- Reset Robustness Testing

  - Inject a reset signal during active operation.

  - Validate that the DUT resets all internal states and restarts correctly.

  - Integration with Test Case: Reset During Operation Test.

- Sparse Data Validation:

  - Provide mostly zero-valued inputs to test efficient handling of sparse datasets.

  - Ensure correct output while avoiding unnecessary MAC operations.

  - Integration with Test Case: Sparse Input Test.

## How This Approach Guarantees Full Coverage?

The testing methodology ensures complete and meaningful validation of the MAC Core by covering all key functional and behavioral aspects observed in realistic and edge-case scenarios. The executed tests provide broad and representative coverage across categories such as correctness, boundary handling, error resilience, and robustness to interruptions.

This strategy integrates complementary types of testing to exercise the DUT under both normal and challenging conditions:

- Randomized Testing - Exploring Broad Input Variability

  - Random combinations of pixel-weight pairs and thresholds simulate diverse operating conditions.

  - Helps uncover unexpected corner cases and increases statistical confidence in DUT correctness.

- Directed Tests - Validating Functional Baselines

  - Confirm correct DUT behavior on well-understood extreme inputs: all-ones and all-zeros.

  - Provide a known-good reference to compare with randomized results.

- Threshold Boundary Testing - Verifying Switching Accuracy

  - Inputs producing sums near the decision threshold validate the DUT’s binary output behavior.

  - Confirms proper functionality at critical decision points.

- Error Handling - Managing Out-of-Range Conditions

  - Supplying invalid thresholds ensures the DUT responds safely and predictably to bad data.

  - Demonstrates robustness against malformed input.

- Interruption and Delay Testing - Ensuring Operational Resilience

  - Pausing data mid-stream or delaying output reading confirms the system remains stable.

  - These tests replicate real-world stalls and ensure no state corruption or output loss.

- Reset Robustness - Validating System Recovery

  - Mid-stream reset checks the ability to restart computation cleanly and from a known state.

  - Critical for long-term stability and recoverability in deployed systems.

- Sparse Data Handling – Testing Efficient Computation

  - Input sets dominated by zero values validate optimized computation and skip logic.

  - Ensures the DUT avoids redundant operations while maintaining correctness.

This layered testing strategy, provides a well-rounded and effective validation of the MAC Core’s external behavior. It ensures the DUT meets expected functionality, handles corner cases gracefully, and remains resilient in real-world operating scenarios. By strategically combining randomness, directed edge values, stress points, and interruption simulations, the test plan delivers confidence in both correctness and robustness of the system.

# Testing plan

The complete testing flow is designed to validate up to 500 test cases per execution cycle using automated test generation, controlled input streaming, and real-time output validation. Each test includes 64 pixel-weight pairs, a 22-bit threshold, and a 1-bit expected output. The process consists of four main stages:

### Detailed Testing Workflow

  - Test Generation Using Python

- A dedicated Python script generates 500 test cases:

- 64 pixel-weight pairs per test (each 16-bit word packs 8-bit pixel and 8-bit weight)

- A 22-bit threshold, split across two 16-bit words

- A single-bit expected output result

- Each test is serialized into a 66-word block (64 + 2), totaling:

- 500 × 66 = 33,000 16-bit words

- A corresponding 500-bit array of expected results is also generated.

- The full memory image (test data + expected outputs) is formatted into a SV file and downloaded into the FPGA’s On-Chip Memory via Quartus tools

- Test Execution on FPGA

- The user presses KEY0, which is debounced and triggers a reset and start pulse.

- The Control Unit asserts start_run, activating the Generator.

- The Generator streams 66 words per test to the DUT:

  - Reads from On-Chip Memory with addr_counter + tests_count × 66

  - Sends them to the DUT via wr_en, address, and chip_sel

- After sending all 66 words, the Generator waits for validation.

- Validation Using the Validator Component

- After receiving all data, the DUT computes the result and asserts output_ready = 1

- The Validator detects output_ready, captures the DUT output, and compares it to the expected output (preloaded from memory).

- It asserts val_done = 1 and sets result = 1 for pass or 0 for fail.

- First and second failures are displayed on HEX0-HEX2 and HEX3-HEX5 respectively. After two failures, testing halts.

#### 4. Result Observation and Reporting

- If two failures occur, testing is paused until the user presses KEY1 to resume.

- Tests continue until all 500 are completed or halted after two failures.

# Test Scenarios

| Test case | Description | Steps | Expected outcome |
| --- | --- | --- | --- |
| 1 | Randomized Input Test | Randomly generate pixel-weight pairs and thresholds. Compare DUT output to a software simulation of the perceptron algorithm. | DUT output matches simulation results. |
| 2 | Maximum Value Test | Input maximum values for pixels, weights, and threshold (all bits set to 1). | DUT handles values correctly without overflow; binary output as expected. |
| 3 | Minimum Value Test | Input minimum values for pixels, weights, and threshold (all bits set to 0). | DUT processes inputs and generates correct outputs. (binary output = 0). |
| 4 | Invalid Threshold Test | Provide an invalid threshold value outside the allowable range. | DUT flags an error or ignores invalid input gracefully. |
| 5 | Threshold Sensitivity Test | Provide inputs that result in cumulative sums close to the threshold (e.g., threshold ±1) and validate that the binary output matches expectations. | The binary output correctly switches as the sum crosses the threshold boundary. |
| 6 | Interrupted Data Flow Test | Stream partial data to the MAC Core, pause the data flow, and then resume input. Validate that the final output is correct despite the interruption. | The MAC Core resumes processing seamlessly and generates the correct result. |
| 7 | Sparse Input Test | Provide mostly zero pixel-weight pairs along with a valid threshold. Validate the intermediate and final results for correctness. | The MAC Core skips unnecessary calculations while maintaining correct results. |
| 8 | Reset During Operation Test | Begin streaming data into the MAC Core and assert the rst_mem signal mid-stream. Resume data input and validate the final output to ensure proper reset. | All internal states reset, and computations restart from an initialized state. |
| 9 | Delayed Output Validation Test | Stream valid inputs to the MAC Core to complete computation, then delay reading the output from the memory. Validate that the stored output remains valid. | The final result remains accurate and accessible despite delays. |

# Detailed Explanations for Test Scenarios

## Randomized Input Test

Objective:
To validate the robustness of the MAC Core by testing it with randomly generated pixel-weight pairs and thresholds.

Importance:
This test ensures the system can handle a wide range of input values, uncovering potential hidden bugs or inconsistencies that may not be caught with directed tests.

Steps:

- Generate a random sequence of pixel-weight pairs.

- Assign a random, valid threshold value.

- Input the data into the MAC Core and monitor the output.

- Compare the results with those from a software simulation of the perceptron algorithm.

## Maximum Value Test

Objective:
To test the MAC Core’s ability to process the largest possible pixel, weight, and threshold values without overflow or errors.

Importance:
Ensures the system remains stable and accurate under extreme conditions, verifying the limits of the arithmetic and memory capabilities.

Steps:

- Set all pixel-weight pairs to their maximum value (255 for an 8-bit system).

- Assign the maximum allowable threshold value.

- Input the data and observe the intermediate and final results.

- Verify that the outputs are correct and there is no overflow or corruption.

## Minimum Value Test

Objective:
To validate the MAC Core’s handling of the smallest possible values (all zeros) for pixels, weights, and thresholds.

Importance:
Ensures the system behaves correctly under "empty" input scenarios, which are common in applications like padding or sparse datasets.

Steps:

- Set all pixel-weight pairs and the threshold to zero.

- Input the data and monitor the intermediate and final outputs.

- Confirm that the binary output is zero and intermediate results are accurate.

## Invalid Threshold Test

Objective:
To test the MAC Core’s behavior when an invalid threshold value is provided.

Importance:
Ensures the system can detect and handle invalid inputs without undefined behavior, which is critical for system robustness.

Steps:

- Provide valid pixel-weight pairs as input.

- Set the threshold to an invalid value (e.g., outside the allowable range or incorrect bit width).

- Observe the system’s reaction and monitor for error flags or graceful handling.

- Confirm the system does not produce undefined outputs.

## Threshold Sensitivity Test

Objective:
To test the system’s ability to correctly classify outputs as binary 1 or 0 based on their proximity to the threshold.

Importance:
Ensures precision in the MAC Core’s decision-making process, especially for results near the threshold boundary.

Steps:

- Provide pixel-weight pairs that result in cumulative sums close to the threshold (±1).

- Set a valid threshold value.

- Monitor the binary output for each case.

- Verify that the output switches correctly as the sum crosses the threshold.

## Interrupted Data Flow Test

Objective:
To evaluate the system’s behavior when the data stream is paused and resumed during operation.

Importance:
Ensures robustness in handling irregular data streams without losing synchronization or accuracy.

Steps:

- Begin streaming valid pixel-weight pairs to the MAC Core.

- Pause the data flow mid-stream and resume after a delay.

- Capture the outputs and verify their correctness.

## Sparse Input Test

Objective:
To validate the system’s efficiency and accuracy when most pixel-weight pairs are zeros.

Importance:
Ensures performance in applications with sparse datasets by skipping unnecessary computations.

Steps:

- Stream mostly zero pixel-weight pairs with a valid threshold.

- Capture intermediate and final results.

- Compare the outputs with expected values to ensure accuracy.

## Reset During Operation Test

Objective:
To test the system’s ability to reset and reinitialize correctly during active computation.

Importance:
Prevents errors or undefined states after unexpected resets, ensuring stability.

Steps:

- Begin streaming data into the MAC Core.

- Lower and then send again the reset signal (chip_sel) mid-operation.

- Resume data streaming and monitor outputs for correctness.

## Delayed Output Validation Test

Objective:
To test the validity of the MAC Core’s output when there is a delay in reading the result after computation.

Importance:
Ensures consistent output retention and accessibility in asynchronous operations.

Steps:

- Stream valid inputs to the MAC Core and complete computation.

- Delay the read operation from the output memory.

- Verify that the stored output remains correct and accessible.